# CIDR (Classless Inter-Domain Routing) — FULL MATH EXPLAINED

## 1️⃣ Why CIDR exists (1-minute context)

Old system = Class A/B/C

- Wasteful (either too big or too small)

- No flexibility

CIDR fixes this by:

- Using prefix length (/24, /26, /30)

- Allowing variable subnet sizes

## 2️⃣ IP address basics (Binary is EVERYTHING)

IPv4 = 32 bits

Example:

192.168.1.10

Binary:

192 → 11000000

168 → 10101000

1   → 00000001

10  → 00001010

So IP =

11000000.10101000.00000001.00001010

## 3️⃣ What /24, /26, /30 actually mean

CIDR notation:

IP / PREFIX

➡️ PREFIX = number of NETWORK bits

### Example: /24

11111111.11111111.11111111.00000000

- Network bits = 24

- Host bits = 32 − 24 = 8

## 4️⃣ Core CIDR MATH FORMULA (memorize this)

### 🔹 Total IPs

2^(host bits)

### 🔹 Usable IPs

2^(host bits) − 2

(network + broadcast are reserved)

## 5️⃣ Common CIDR blocks (MUST KNOW)

| CIDR | Host bits | Total IPs | Usable IPs |
| --- | --- | --- | --- |
| /24 | 8 | 256 | 254 |
| /25 | 7 | 128 | 126 |
| /26 | 6 | 64 | 62 |
| /27 | 5 | 32 | 30 |
| /28 | 4 | 16 | 14 |
| /29 | 3 | 8 | 6 |
| /30 | 2 | 4 | 2 |
| /32 | 0 | 1 | 1 |

👉 AWS VPC note: AWS reserves 5 IPs, not 2.

## 6️⃣ Subnet Mask math (CIDR → mask)

### Trick: Fill 1s from left

#### /26

11111111.11111111.11111111.11000000

Convert last octet:

11000000 = 128 + 64 = 192

➡️ Subnet mask:

255.255.255.192

## 7️⃣ Block size formula (FAST subnetting)

Block Size = 256 − subnet mask octet

### Example: /26

256 − 192 = 64

So networks are:

0, 64, 128, 192

## 8️⃣ Full subnet calculation (step-by-step)

### Given:

192.168.1.70/26

### Step 1: Block size

256 − 192 = 64

### Step 2: Network ranges

0–63

64–127

128–191

192–255

### Step 3: Find where 70 fits

64–127

### Final answers:

Network ID     → 192.168.1.64

First usable   → 192.168.1.65

Last usable    → 192.168.1.126

Broadcast      → 192.168.1.127

## 9️⃣ Supernetting (reverse CIDR math)

### Example:

Combine:

192.168.0.0/24

192.168.1.0/24

Total IPs:

256 + 256 = 512

Closest power of 2:

512 = 2^9

Prefix:

32 − 9 = /23

➡️ Result:

192.168.0.0/23

## 🔟 CIDR in AWS (exam-critical)

### VPC rules:

- Min: /28

- Max: /16

- Private ranges:

  - 10.0.0.0/8

  - 172.16.0.0/12

  - 192.168.0.0/16

### AWS reserves 5 IPs per subnet:

.0 → Network

.1 → Router

.2 → DNS

.3 → Reserved

Last → Broadcast

So /24 usable in AWS:

256 − 5 = 251

## 11️⃣ CIDR cheat patterns (mental math)

### Want ~50 hosts?

2^6 = 64 → /26

### Want ~100 hosts?

2^7 = 128 → /25

### Point-to-point link?

/30

## 12️⃣ One-page CIDR exam cheat sheet 🧠

Hosts = 2^(32 − prefix) − 2

Block = 256 − mask

Subnet jump = block size